SECTION 09 77 00
SPECIAL WALL SURFACING

GENERAL

SUMMARY

Section Includes:   Durable, decorative wall panels with smooth or textured surfaces adhered to unfinished [gypsum] [untreated plywood] wallboard. Adhesives, accessories, and trims.

[Aluminum trim.]

Products Not Furnished or Installed under This Section:

Gypsum [Cementitious] substrate board.

Resilient Base.

RELATED SECTIONS

Drawings and general provisions of the Contract, including General and Supplementary Conditions and other Division 01 Specification Sections, apply to this Section.

Section 09 29 00 – Gypsum [Cementitious] substrate board

Section 06 11 00 - Wood [Metal] Stud Framing

Section 09 90 00 - Painting & Transparent Finishes

Section 09 65 13 - Resilient Base

REFERENCES

References are listed as a convenience and are often deleted in their entirety.  If required by the user agency delete references from the list below that are not actually required by the text of the edited section.

American Society for Testing and Materials:  Standard Specifications (ASTM)

ASTM D 256 – Izod Pendulum Impact Resistance of Plastics.

ASTM D 638 – Tensile Properties of Plastics.

ASTM D 790 – Flexural Properties of Unreinforced and Reinforced Plastic and Electrical Insulating Materials.

ASTM D 2583 – Indentation Hardness of Rigid Plastics by Means of a Barcol Impressor

SUBMITTALS

Product Data: Submit sufficient manufacturer's data to indicate compliance with these specifications, including:

Preparation instructions and recommendations.

Storage and handling requirements and recommendations.

Installation methods.

Shop Drawings: Submit elevations of each wall showing location of paneling and trim members with respect to all discontinuities in the wall elevation.

Selection Samples: Submit sample of required color or pattern selected by architect.

Samples for Verification:   Submit appropriate section of panel for each finish selected indicating the color / print and finish required.

Submit complete with specified applied finish.

Exposed Molding and Trim: Provide samples of each type, finish, and color.

Manufacturers Material Safety Data Sheets (MSDS) for adhesives, sealants and other pertinent materials prior to their delivery.

QUALITY ASSURANCE

Conform to building code requirements for interior finish for smoke and flame spread requirements as tested in accordance with:

ASTM E 84 (Method of test for surface burning characteristics of building Materials)

            - Wall Required Rating – Class C.

- Include the following if products are used in areas subject to stringent wash down requirements.

Regulatory requirements and approvals: [Specify applicable requirements of regulatory agencies].

DELIVERY, STORAGE AND HANDLING

Deliver materials factory packaged on strong pallets.

Store panels and trim lying flat, under cover and protected from the elements.  Allow panels to acclimate to room temperature (range of 60 to 75°F) for 48 hours prior to installation.

PROJECT CONDITIONS

Environmental Limitations: Buildings are to be fully enclosed prior to installation with sufficient heat (70°) and ventilation consistent with good working conditions for finish work.

During installation and for not less than 48 hours before, maintain an ambient temperature and relative humidity within limits required by type of adhesive used and recommendation of adhesive manufacturer.

Provide ventilation to disperse fumes during application of adhesive as recommended by the adhesive manufacturer.

WARRANTY

Furnish three-year guarantee against defects in material and workmanship.

PRODUCTS

ACCEPTABLE MANUFACTURER

Composite Wall Panels Basis-of-Design: Marlite Induro™ Aluminum Composite Material (ACM) Panels manufactured by Marlite, www.marlite.com. For questions or clarification, contact Marli Harrison at 800.377.1221 or email: marketing@marlite.com.

The Architect will consider products of comparable manufacturers as a substitution, pending the contractor’s submission of complete details and documentation of the substitution in accordance with procedures in Division 01 of the Project Manual. Requests for alternates must be submitted a minimum of 10 days prior to bid. Documentation shall include a list of five similar projects of equivalent size where products have been installed for a minimum of two years.

PANELS

Induro™ ACM Panels:  The surface face material is High Pressure Laminate (HPL).  The core is an aluminum composite material (ACM) with a High Density Polyethylene (HDPE) center.  A balancing backer is applied for panel stability.

Surface:

            - High pressure laminate (HPL): [Manufacturer] [Manufacturer’s color and product number]

Dimensions:

            - Thickness – 0.150” (3.8mm) nominal

            - Size – [4 feet x 8 feet (1.2 x 2.4m)] [4 feet x 10 feet (1.2 x 3.0m)] Induro FRP Panels are not available in lengths exceeding 8’ (2.4m).

Tolerance:

            - Length and Width:  +/-1/8” (3.175mm)

Testing Properties:

            - Must meet all standards as listed in Paragraph 1.3.A.

Fire Rating:

Class C Fire Rating.

- Delete the following if resilient or contractor supplied bases are used.

MOLDINGS

Division, Inside, Outside, and Edge Molding:

Aluminum Anodized Trim: Heavy weight extruded aluminum 6063-T5 alloy prefinished at the factory.

            - Color: [Brite Satin Anodized] [Black Powder Coat] [Grey Powder Coat] [Brown Powder Coat]

            - Length: [8’] [10’]

Corner Guard:

F 560SS #4 Brushed Satin Stainless Corner Guard

Accessories

- Specify rivets for use in areas with dramatic changes in temperature and/or humidity, where the substrate is unusually uneven, and in all low temperatures or cold storage applications.

Adhesives Available Required by Substrate:

Titebond® Advanced Polymer Panel Adhesive – VOC compliant, non-flammable, environmentally safe adhesive.

Sealant:

Marlite Brand MS-250 Clear Silicone Sealant.

EXECUTION

PREPARATION

Examine areas and conditions, with Installer present, to ensure compliance that corners are plumb and straight, surfaces are smooth, uniform, clean and free from foreign matter, nails countersunk, joints and cracks filled flush and smooth with the adjoining surface. If conditions are not acceptable, inform the General Contractor immediately in writing.

Verify that stud spacing does not exceed 24” (61cm) on-center.

Level wall surfaces to panel manufacturer’s requirements.  Remove protrusions and fill indentations.

Proceed with installation only after unsatisfactory conditions have been corrected.

INSTALLATION

Comply with manufacturer's recommended procedures and installation sequence.

Cut sheets to meet supports allowing 1/8” (3 mm) clearance for every 8 feet (2.4m) of panel.

Cut and drill with carbide tipped saw blades or drill bits or cut with shears.

For cut-outs for utilities, round all corners.

Apply panels to board substrate, above base, vertically oriented with seams plumb and pattern aligned with adjoining panels.

Install panels with manufacturers’ recommended gap for panel field and corner joints.

            - Adhesive trowel and application method to conform to adhesive manufacturer’s recommendations.

Apply panel moldings to all panel edges using silicone sealant providing for required clearances.

All moldings must provide for a minimum 1/8” (3mm) of panel expansion at joints and edges, to ensure proper installation.

CLEANING

Remove excess adhesive and sealant from panels and molding immediately.  Wipe panel down using a damp cloth and mild soap solution or cleaner.

Refer to manufacturer's specific cleaning recommendations. Do not use abrasive cleaners.

END OF SECTION